Section II: Schedule of Requirements

Supply, testing, delivery and unloading of HDPE Pipes and Fittings to Warehouses of Taiz City Local Water and Sanitation Corporation/Authority–, Taiz City - Yemen.

LOT 1 - Supply, testing, delivery and unloading of HDPE Pipes and Fittings to Warehouses of Taiz City Local Water and Sanitation Corporation/Authority, Taiz City - Yemen.

- Summary of requirements

| Subproject ID | Description | Destination City |
| --- | --- | --- |
| EHC-AF3-WS-TAZ-018-01 | Supply of HDPE Pipes and Fittings | Taiz City |

## Technical specifications for Goods and Comparative Data Table

| ITEM#1: HDPE Pipes: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | UNOPS minimum technical requirements |  | Bidder to complete | Bidder to complete |
| 1. 1 | Date of manufacture: New (2025) |  | ☐ Yes ☐ No |  |
| 1. 2 | Manufacturer/Brand |  | To be inserted by bidder |  |
| 1. 3 | Country of origin: |  | To be inserted by bidder |  |
| 1. 4 | Pressure Rating: | SDR 11 (PN16) | ☐ Yes ☐ No |  |
| 1.4.1 | Diameter Range | DN 25 mm – DN 110 mm Please refer to additional technical requirement. | ☐ Yes ☐ No |  |
| 1. 5 | HDPE pressure Pipes | -100% virgin (with no recycled compound) PE. 100 | ☐ Yes ☐ No |  |
|  |  | -MRS ≥ 10 MPa |  |  |
|  |  | -Density 0.95–0.965 g/cm³ |  |  |
|  |  | -Physical Properties: Melt flow index 0.4–0.55 g/10min; tensile ≥ 25 N/mm²; elongation ≥ 600%; modulus 1200 N/mm² |  |  |
| 1. 6 | The standards for PE Pipes shall be Comply with current international standards or equivalents: |  | ☐ Yes ☐ No |  |
|  | ● DIN EN ISO 178 |  |  |  |
|  | ● DIN EN ISO 527-2 |  |  |  |
|  | ● DIN 7728-1 |  |  |  |
|  | ● DIN 8074 / ISO 4427 |  |  |  |
|  | ● DIN 8075 / ISO 4427 |  |  |  |
|  | ● DIN EN ISO 12162 |  |  |  |
|  | ● DIN 53752 |  |  |  |
|  | ● EN 12201 |  |  |  |
| 1. 7 | ● Use: Municipal and urban water supply and distribution systems. |  | ☐ Yes ☐ No |  |
|  | ● Compliance certificates verifying potable water suitability (WRAS, NSF, BS 6920, or equivalent). |  |  |  |
| 1. 8 | The bidder confirms that manufacturer quality-control test results and reports, as well as batch-traceability documents, will be provided at the delivery stage. |  | ☐ Yes ☐ No |  |
| 1. 9 | Packaging & Supply Format: Pipes with a nominal diameter of DN 110 mm or smaller shall be supplied in coiled form, with a maximum coil length of 100 meters, DN>110 mm straight |  | ☐ Yes ☐ No |  |
| 1. 10 | Certifications: Manufacturer shall hold valid ISO 9001, ISO 14001, OHSAS 18001 (or equivalent) certifications. |  | ☐ Yes ☐ No |  |
| 1.11 | Marking & traceability per ISO/DIN standards |  | ☐ Yes ☐ No |  |
| ITEM # 2: PP Compression Fittings (Couplings, Reducers, Tees, Adaptors, Elbows, End Caps) |  |  |  |  |
| Item | UNOPS minimum technical requirements |  | Bidder to complete | Bidder to complete |
| 2. 1 | Date of manufacture | 2025 or newer | To be inserted by bidder |  |
| 2. 2 | Manufacturer |  | To be inserted by bidder |  |
| 2. 3 | Country of origin |  | To be inserted by bidder |  |
| 2. 4 | Pressure Rating: | PN16 (16 bar) and PN (10) Please refer to additional technical requirement. | ☐ Yes ☐ No |  |
| 2. 5 | Types and SIZES | Please refer to additional technical requirement. | ☐ Yes ☐ No |  |
| 2. 6 | - Material: PP-B (UV stabilized); EPDM seals; POM grip rings; stainless steel reinforcement |  | ☐ Yes ☐ No |  |
|  | Pressure Class: PN16 and PN10, please refer to additional technical requirement. |  |  |  |
|  | - Threads BSP in accordance with ISO 7-1 / EN 10226 |  |  |  |
|  | - Standards: EN ISO 17885, EN 12201-3, EN 681-1or equivalent |  |  |  |
|  | - Potable water suitability certificates |  |  |  |
|  | - Marking & batch traceability |  |  |  |
| 2. 7 | - Pressure Class: PN16 (PN10 for some end caps as indicated in Form Bs) |  | ☐ Yes ☐ No |  |
| 2. 8 | - Threads BSP in accordance with ISO 7-1 / EN 10226 |  | ☐ Yes ☐ No |  |
| 2. 9 | - Standards: EN ISO 17885, EN 12201-3, EN 681-1 or equivalent |  | ☐ Yes ☐ No |  |
| 2. 10 | - Potable water suitability certificates |  | ☐ Yes ☐ No |  |
| ITEM # 3: HDPE Butt Fusion Fittings |  |  |  |  |
| Item | UNOPS minimum technical requirements |  | Bidder to complete | Bidder to complete |
| 3.1 | Date of manufacture | 2025 or newer | To be inserted by bidder |  |
| 3.2 | - Manufacturer |  | To be inserted by bidder |  |
| 3.3 | - Country of origin |  | To be inserted by bidder |  |
| 3.4 | - Material: PE100, 100% virgin |  | ☐ Yes ☐ No |  |
|  | - Pressure Class: SDR11, PN16 |  |  |  |
|  | - Types & Sizes: please refer to Additional Tech Requirements |  |  |  |
|  | - Standards: EN 1555-3, EN 12201-3, ISO 13953 or equivalent |  |  |  |
|  | - Potable water suitability |  |  |  |
|  | - HDPE Fittings for butt fusion systems - Type: Fittings are injection molded, Long Version (long spigot ends) - Fabricated or segmented fittings are not accepted. |  |  |  |
|  | - Marking & traceability |  |  |  |
| 3.5 | Steel Backing Flange - Material: Galvanized mild steel S235JR toto EN 10025-2, or ASTM A36, or approved equivalent. - Connecting dimensions: ISO 7005, EN 1092, DIN 2501 - Bolt circle: PN 16 - Each flange shall be supplied with gaskets, bolts, nuts, and washers in stainless steel grade A2/304. |  | ☐ Yes ☐ No |  |
| ITEM # 4: Restraint Couplings & Flange Adaptors |  |  |  |  |
| Item | UNOPS minimum technical requirements |  | Bidder to complete | Bidder to complete |
| 4.1 | - Date of manufacture | 2025 or above | ☐ Yes ☐ No |  |
| 4.2 | - Manufacturer |  | To be inserted by bidder |  |
| 4.3 | - Country of origin |  | To be inserted by bidder |  |
| 4.4 | - Standards: EN 12842 or equivalent |  | To be inserted by bidder |  |
|  | - Working Pressure:PN16 |  |  |  |
|  | - Flange Compatibility: EN1092-2or equivalent |  |  |  |
|  | - Angular Deflection: ≥ 1.5° |  |  |  |
|  | Sizes and types: Please refer to additional technical requirement. |  |  |  |
|  | Material: |  |  |  |
|  | - End Ring: Ductile Iron, FBE Coated |  |  |  |
|  | - Grip Ring: Copper Alloy |  |  |  |
|  | - Bolts :316 Stainless Steel with Delta Seal Coating |  |  |  |
|  | - Elastomeric Seals: EPDM |  |  |  |
|  | - Barrel: Ductile Iron, FBE Coated |  |  |  |
| Others |  |  |  |  |
| 1 | Warranty: minimum 1-year against manufacturing defects, leakage, or premature failure from the handover certificate date |  | ☐ Yes ☐ No |  |

| ITEM#1: HDPE Pipes: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | UNOPS minimum technical requirements |  | Bidder to complete | Bidder to complete |
| 1. 1 | Date of manufacture: New (2025) |  | ☐ Yes ☐ No |  |
| 1. 2 | Manufacturer/Brand |  | To be inserted by bidder |  |
| 1. 3 | Country of origin: |  | To be inserted by bidder |  |
| 1. 4 | Pressure Rating: | SDR 11 (PN16) | ☐ Yes ☐ No |  |
| 1.4.1 | Diameter Range | DN 25 mm – DN 110 mm Please refer to additional technical requirement. | ☐ Yes ☐ No |  |
| 1. 5 | HDPE pressure Pipes | -100% virgin (with no recycled compound) PE. 100 | ☐ Yes ☐ No |  |  |
|  |  | -MRS ≥ 10 MPa |  |  |  |
|  |  | -Density 0.95–0.965 g/cm³ |  |  |  |
|  |  | -Physical Properties: Melt flow index 0.4–0.55 g/10min; tensile ≥ 25 N/mm²; elongation ≥ 600%; modulus 1200 N/mm² |  |  |  |
| 1. 6 | The standards for PE Pipes shall be Comply with current international standards or equivalents: |  | ☐ Yes ☐ No |  |  |
|  | ● DIN EN ISO 178 |  |  |  |  |
|  | ● DIN EN ISO 527-2 |  |  |  |  |
|  | ● DIN 7728-1 |  |  |  |  |
|  | ● DIN 8074 / ISO 4427 |  |  |  |  |
|  | ● DIN 8075 / ISO 4427 |  |  |  |  |
|  | ● DIN EN ISO 12162 |  |  |  |  |
|  | ● DIN 53752 |  |  |  |  |
|  | ● EN 12201 |  |  |  |  |
| 1. 7 | ● Use: Municipal and urban water supply and distribution systems. |  | ☐ Yes ☐ No |  |  |
|  | ● Compliance certificates verifying potable water suitability (WRAS, NSF, BS 6920, or equivalent). |  |  |  |  |
| 1. 8 | The bidder confirms that manufacturer quality-control test results and reports, as well as batch-traceability documents, will be provided at the delivery stage. |  | ☐ Yes ☐ No |  |  |
| 1. 9 | Packaging & Supply Format: Pipes with a nominal diameter of DN 110 mm or smaller shall be supplied in coiled form, with a maximum coil length of 100 meters, DN>110 mm straight |  | ☐ Yes ☐ No |  |  |
| 1. 10 | Certifications: Manufacturer shall hold valid ISO 9001, ISO 14001, OHSAS 18001 (or equivalent) certifications. |  | ☐ Yes ☐ No |  |  |
| 1.11 | Marking & traceability per ISO/DIN standards |  | ☐ Yes ☐ No |  |  |
| ITEM # 2: PP Compression Fittings (Couplings, Reducers, Tees, Adaptors, Elbows, End Caps) |  |  |  |  |  |
| Item | UNOPS minimum technical requirements |  | Bidder to complete | Bidder to complete |  |
| 2. 1 | Date of manufacture | 2025 or newer | To be inserted by bidder |  |  |
| 2. 2 | Manufacturer |  | To be inserted by bidder |  |  |
| 2. 3 | Country of origin |  | To be inserted by bidder |  |  |
| 2. 4 | Pressure Rating: | PN16 (16 bar) and PN (10) Please refer to additional technical requirement. | ☐ Yes ☐ No |  |  |
| 2. 5 | Types and SIZES | Please refer to additional technical requirement. | ☐ Yes ☐ No |  |  |
| 2. 6 | - Material: PP-B (UV stabilized); EPDM seals; POM grip rings; stainless steel reinforcement |  | ☐ Yes ☐ No |  |  |
|  | Pressure Class: PN16 and PN10 , Please refer to additional technical requirement. |  |  |  |  |
|  | - Threads BSP in accordance with ISO 7-1 / EN 10226 |  |  |  |  |
|  | - Standards: EN ISO 17885, EN 12201-3, EN 681-1or equivalent |  |  |  |  |
|  | - Potable water suitability certificates |  |  |  |  |
|  | - Marking & batch traceability |  |  |  |  |
| 2. 7 | - Pressure Class: PN16 (PN10 for some end caps as indicated in Form Bs) |  | ☐ Yes ☐ No |  |  |
| 2. 8 | - Threads BSP in accordance with ISO 7-1 / EN 10226 |  | ☐ Yes ☐ No |  |  |
| 2. 9 | - Standards: EN ISO 17885, EN 12201-3, EN 681-1 or equivalent |  | ☐ Yes ☐ No |  |  |
| 2. 10 | - Potable water suitability certificates |  | ☐ Yes ☐ No |  |  |
| ITEM # 3: HDPE Butt Fusion Fittings |  |  |  |  |  |
| Item | UNOPS minimum technical requirements |  | Bidder to complete | Bidder to complete |  |
| 3.1 | Date of manufacture | 2025 or newer | To be inserted by bidder |  |  |
| 3.2 | - Manufacturer |  | To be inserted by bidder |  |  |
| 3.3 | - Country of origin |  | To be inserted by bidder |  |  |
| 3.4 | - Material: PE100, 100% virgin |  | ☐ Yes ☐ No |  |  |
|  | - Pressure Class: SDR11, PN16 |  |  |  |  |
|  | Types & Sizes: Please refer to additonal technical requriment. |  |  |  |  |
|  | - Standards: EN 1555-3, EN 12201-3, ISO 13953 or equivalent |  |  |  |  |
|  | - Potable water suitability |  |  |  |  |
|  | - Marking & traceability |  |  |  |  |
| ITEM # 4: Restraint Couplings & Flange Adaptors |  |  |  |  |  |
| Item | UNOPS minimum technical requirements |  | Bidder to complete | Bidder to complete |  |
| 4.1 | - Date of manufacture | 2025 or above | ☐ Yes ☐ No |  |  |
| 4.2 | - Manufacturer |  | To be inserted by bidder |  |  |
| 4.3 | - Country of origin |  | To be inserted by bidder |  |  |
| 4.4 | - Standards: EN 12842 or equivalent |  | To be inserted by bidder |  |  |
|  | - Working Pressure:PN16 |  |  |  |  |
|  | - Flange Compatibility: EN1092-2or equivalent |  |  |  |  |
|  | - Angular Deflection: ≥ 1.5° |  |  |  |  |
|  | Sizes and types: Please refer to additional technical requirement. |  |  |  |  |
|  | Material: |  |  |  |  |
|  | - End Ring: Ductile Iron, FBE Coated |  |  |  |  |
|  | - Grip Ring: Copper Alloy |  |  |  |  |
|  | - Bolts :316 Stainless Steel with Delta Seal Coating |  |  |  |  |
|  | - Elastomeric Seals: EPDM |  |  |  |  |
|  | - Barrel: Ductile Iron, FBE Coated |  |  |  |  |
| Others |  |  |  |  |  |
| 1 | Warranty: minimum 1-year against manufacturing defects, leakage, or premature failure from the handover certificate date |  | ☐ Yes ☐ No |  |  |

- Delivery requirements – Comparative Data Table:

| UNOPS Requirements |  | Is quotation compliant? Bidder to complete | Details Bidder to complete |
| --- | --- | --- | --- |
| Delivery time | The Bidder shall deliver the goods within 6 months after the Contract signature. | ☐ Yes ☐ No | Insert details |
| Delivery place and Incoterms rules | DAP as per Incoterms 2020 at Warehouses of National/Local Water & Sanitation Authorities at Taiz City – Yemen | ☐ Yes ☐ No | Insert details |
| UNOPS Right to vary requirements | At the time the Contract is awarded, UNOPS reserves the right to vary the quantity of the goods and associated services specified above, provided this does not exceed +/- [20%] , without any change in the unit prices or other terms and conditions of the RFQ. | ☐ Yes ☐ No | Insert details |